Department of Electrical and Electronic Engineering

Mymensingh Engineering College

Project and Thesis: EEE4100

Design, Simulation, Implementation, and Digital Twin Integration of a
Thrust Vector Control System and Telemetry System for a Small-Scale Rocket
Using Adaptive Control Methods

Supervisor: Asst. Prof. Rabeya Akter
Department of Electrical and Electronic Engineering
Mymensingh Engineering College, Mymensingh

Authors:

Lian Mollick Nehal   |   Reg: 2835   |   Roll: 1253   |   Exam Roll: 3529

Oliul Islam Rian   |   Reg: 2833   |   Roll: 1251   |   Exam Roll: 3528

Sadman Al Mahmud   |   Reg: 2805   |   Roll: 1222   |   Exam Roll: 3536

ABSTRACT

Flight stability is a critical requirement for solid-propellant rocket systems, where small deviations in attitude can result in trajectory divergence and mission failure. This thesis presents the complete design, simulation, hardware prototyping, and digital twin integration of a Thrust Vector Control (TVC) system for a small-scale rocket (Project Neal) powered by an ammonium perchlorate composite propellant (APCP) motor. A gimbaled nozzle mechanism actuated by two micro-servomotors provides active pitch and yaw control. A three-degrees-of-freedom (3-DOF) and a six-degrees-of-freedom (6-DOF) rigid-body dynamics model were developed in MATLAB/Simulink, incorporating aerodynamic coefficients from OpenRocket, experimentally derived mass moments of inertia, and motor thrust curves from OpenMotor and thrustcurve.org. Four feedback controllers are designed and benchmarked under 500-run Monte Carlo mass uncertainty and wind perturbation scenarios: classical PID, linear-quadratic-integral (LQI), model-reference adaptive control (MRAC), and active disturbance rejection control (ADRC). ADRC achieves zero overshoot and 99.8% mission success rate without requiring an accurate plant model. Hardware was implemented on an STM32F103C8T6 microcontroller with an MPU6050 IMU, BME280 barometric sensor, and LoRa-based telemetry link. A hardware-in-the-loop digital twin was developed as a browser-based React/Three.js dashboard with a FastAPI telemetry bridge, enabling real-time 6-DOF visualization, interactive TVC simulation, and live sensor monitoring. Bench testing confirmed responsive, stable gimbal actuation closely matching simulated performance.

Keywords: Thrust Vector Control, ADRC, Digital Twin, Model Rocketry, 3-DOF, 6-DOF, PID, LQI, MRAC, STM32, Hardware-in-the-Loop, Kalman Filter, APCP, OpenRocket.

ABBREVIATIONS

ADRC    Active Disturbance Rejection Control

ACS     Altitude Control System

APCP    Ammonium Perchlorate Composite Propellant

CG      Center of Gravity

CP      Center of Pressure

DOF     Degrees of Freedom

ESO     Extended State Observer

FC      Flight Computer

GS      Ground Station

HIL     Hardware-in-the-Loop

IMU     Inertial Measurement Unit

ISR     Interrupt Service Routine

LQI     Linear-Quadratic-Integral

LQR     Linear Quadratic Regulator

MOI     Moment of Inertia

MRAC    Model-Reference Adaptive Control

PID     Proportional-Integral-Derivative

RCS     Reaction Control System

TVC     Thrust Vector Control

UART    Universal Asynchronous Receiver-Transmitter

I.  INTRODUCTION

Stability and control are fundamental requirements for the successful flight of any rocket system, regardless of scale. During the powered ascent phase, even small deviations in attitude can result in trajectory divergence, aerodynamic instability, or complete mission failure. For small-scale solid-propellant rockets, this challenge is amplified by low overall mass, high thrust-to-weight ratio, and short burn times that leave minimal margin for correction. Manufacturing tolerances, asymmetries in fin alignment, and wind disturbances can introduce angular deviations that quickly escalate into unstable flight [3].

Thrust Vector Control (TVC) is one of the most effective active stabilization techniques in rocketry. By dynamically altering the thrust axis, TVC directly influences the angular acceleration of the rocket, enabling rapid attitude corrections during powered flight [4]. While TVC has long been implemented in large-scale aerospace vehicles using hydraulic actuators or electromechanical servos, scaling these systems to lightweight educational rockets presents unique challenges in actuator speed, control precision, mechanical robustness, and onboard processing. The Putimach launch by DhumketuX on February 7, 2023, marked Bangladesh's first entry into experimental rocketry [5], but its passive fin-stabilized design exhibited erratic flight trajectories, highlighting the critical need for active guidance systems.

This thesis bridges the gap between simulation-based control design and functional hardware implementation. In addition to the physical prototype, a real-time hardware-in-the-loop (HIL) digital twin is developed as a browser-based dashboard, enabling interactive simulation, live telemetry visualization, and controller benchmarking before live flight. This unified workflow — from dynamic modeling to embedded control to digital twin integration — represents the primary contribution of this work.

1.1  Objectives

Design a stable aerodynamic configuration using OpenRocket, determining stability margin, CG, and CP.

Develop 3-DOF and 6-DOF rigid-body dynamics models in MATLAB/Simulink with aerodynamic, gravitational, and thrust forces.

Design, implement, and benchmark four TVC controllers: PID, LQI, MRAC, and ADRC.

Fabricate a lightweight 3D-printed gimbaled nozzle mount with MG90 digital servo actuation.

Build a flight control computer (FCC) on STM32F103C8T6 with MPU6050, BME280, and LoRa telemetry.

Implement Kalman filter sensor fusion for real-time attitude estimation.

Develop a hardware-in-the-loop digital twin with 6-DOF visualization and live telemetry integration.

Validate system performance through Monte Carlo simulation and benchtop hardware testing.

1.2  Problem Statement

Existing research frequently treats design, simulation, and prototype testing of TVC rockets as separate exercises, slowing iteration and hiding integration problems such as sensor-actuator timing, gimbal packaging, and thermal exposure. Low-cost, repeatable workflows that tightly couple vehicle design (CG/CP), dynamic simulation (6-DOF plus actuator models), and physical prototyping with real-time digital twin monitoring are scarce in the literature despite clear demand from university teams and low-cost launcher developers [6].

II.  THEORETICAL BACKGROUND

2.1  Rocket Stability: CG and CP

Passive stability requires the Center of Pressure to be located aft of the Center of Gravity. When the angle of attack changes, aerodynamic forces on the fins create a restoring torque around the CG. However, this passive stability is only effective at velocities where aerodynamic forces are significant. During the initial low-speed ascent, the rocket is inherently unstable — the primary problem that active TVC is designed to solve [26].

2.2  Thrust Vector Control Methods

TVC achieves attitude control by gimbaling the engine nozzle to redirect thrust. Methods documented in NASA-SP-8114 include: flexible joint, gimbaled nozzle, ball-in-socket, secondary fluid injection (liquid or gas), jet vanes, jet tabs, and jetevators [4,10,11]. This project uses a gimbaled nozzle mechanism with a two-axis servo-actuated gimbal, providing pitch and yaw control authority of plus and minus 5 degrees.

2.3  APCP Propulsion and Grain Geometry

The G74W motor uses ammonium perchlorate composite propellant (APCP) with hydroxyl-terminated polybutadiene (HTPB) binder. Internal ballistic simulation predicts a peak chamber pressure of 73 bar, average thrust 74 N, burn time 1.12 s, and total impulse 90 N·s. Fuel grain regression analysis shows a near-linear port radius increase at 20.19 mm/s (R² = 0.899), indicating stable, predictable combustion suitable for a controlled TVC burn [14].

2.4  Sensor Fusion: Kalman Filter

The MPU6050 gyroscope measures angular velocity but accumulates drift when integrated. The accelerometer measures tilt accurately in steady state but is noisy during powered flight. A discrete Kalman filter fuses both sensors in a predict-correct cycle at 200 Hz, producing a smooth, drift-corrected attitude estimate. The filter state is [theta, theta_dot] with process noise Q = diag(0.001, 0.003) and measurement noise R = 0.03 [17].

III.  VEHICLE MODEL AND PARAMETERS

Table I summarizes the inertial and propulsion parameters of Project Neal 1.2 as derived from OpenRocket and hardware measurement.

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Wet launch mass | m_wet | 2.055 | kg |
| Dry mass | m_dry | 1.968 | kg |
| Propellant mass | m_p | 0.087 | kg |
| Average thrust | F_t | 74–91.2 (peak) | N |
| Burn time | t_b | 1.12 | s |
| Total impulse | I_t | 90.0 | N·s |
| Pitch MOI (wet) | I_yy | 0.028 | kg·m² |
| Roll MOI | I_xx | 0.003 | kg·m² |
| Yaw MOI | I_zz | 0.028 | kg·m² |
| Nozzle-CG arm | l_cm | 0.22 | m |
| Gimbal limit | delta_max | ±5.0 | deg |
| Body length | L | 0.68 | m |
| Body diameter | D | 0.038 | m |
| Stability margin | SM | 2.06 | cal |
| CG location | x_CG | 63.4 | cm from nose |
| CP location | x_CP | 79.1 | cm from nose |
| Apogee (sim) | h_ap | 59.7 | m |
| Max velocity (sim) | V_max | 31.5 | m/s |

Table I — Project Neal 1.2 Inertial, Aerodynamic, and Propulsion Parameters

3.1  3-DOF Pitch-Plane Equations of Motion

The pitch-plane (3-DOF) model captures rotational dynamics during the propulsion phase. The state vector is x = [theta, theta_dot, x_dot]^T representing pitch angle, pitch rate, and lateral drift velocity. The state-space representation is x_dot = Ax + Bu, where:

theta_double_dot = (F_t * l_cm * sin(delta) - D * z_cp) / I_yy   (1)

z_double_dot = (F_t * cos(delta) - m*g) / m   (2)

x_double_dot = (F_t * sin(delta) - D * sin(alpha)) / m   (3)

System matrices A and B are populated using aerodynamic and physical data exported from OpenRocket. The simulation is discretized at dt = 0.01 s using a fourth-order Runge-Kutta integrator [29].

3.2  6-DOF Full Rotational Model

The 6-DOF model adds roll and yaw channels with aerodynamic damping and cross-coupling disturbances, implemented in the digital twin backend:

I_yy * q_dot = F * l_cm * sin(delta_p) - b_q * q   (4)

I_xx * p_dot = -b_p * p + d_phi   (roll, uncontrolled)   (5)

I_zz * r_dot = F * l_cm * sin(delta_y) - b_r * r   (6)

where p, q, r are roll-pitch-yaw rates; delta_p, delta_y are gimbal deflections; b_q, b_r = 0.4 N·m·s, b_p = 0.3 N·m·s are damping coefficients; d_phi is external roll disturbance torque (non-zero in wind gust scenario).

IV.  CONTROLLER DESIGNS

4.1  PID Controller

A classical PID controller is tuned at the nominal mid-burn operating point. The control law is:

delta(t) = K_p * e(t) + K_i * integral(e) + K_d * e_dot(t)   (7)

In the MATLAB simulation, tuning followed Ziegler-Nichols methodology with manual refinement. Final gains: K_p = 0.2109, K_i = 0.19, K_d = 0.05 (simulation). Hardware gains for 50 Hz embedded loop: K_p = 4.2, K_i = 0.8, K_d = 0.35. Integrator anti-windup via back-calculation (T_t = 0.1 s). Gimbal rate limited to 120 deg/s to prevent actuator saturation [30].

Design requirements met: overshoot < 10%, settling time < 1.5 s, rise time < 2 s, steady-state error < 2%. Simulated response to a 5-degree step disturbance shows settling at the setpoint in under 1.5 s with approximately 5.5-degree peak (5% overshoot).

4.2  Linear-Quadratic-Integral (LQI) Controller

The LQI regulator augments the state with the integral of pitch error to eliminate steady-state offset. The continuous-time cost functional is:

J = integral_0_inf (x^T Q x + u^T R u) dt   (8)

State weighting: Q = diag(10, 1, 5, 0.1); input weight: R = 0.01. Gain matrix K is computed via the discrete algebraic Riccati equation and discretized at 50 Hz via zero-order hold. LQI achieves the lowest mean pitch RMSE (1.89 deg) but introduces 7.8% overshoot due to the integral action during the transient.

4.3  Model-Reference Adaptive Control (MRAC)

MRAC adjusts the control gain online to force the plant output to track a stable reference model with bandwidth omega_m = 12 rad/s, damping zeta_m = 0.9. The MIT adaptation rule is:

K_dot(t) = -gamma * e(t) * theta(t)   (9)

Adaptation gain gamma = 0.5. The adaptation transient (0.55 s) exceeds the burn time, resulting in 30.4% overshoot — the highest of all tested controllers. MRAC is not recommended for this rocket class where the adaptation window is shorter than the total control authority window.

4.4  Active Disturbance Rejection Control (ADRC)

ADRC treats all unmodeled dynamics, parameter variations, and external disturbances as a single lumped total disturbance f(t) estimated in real time by a second-order extended state observer (ESO) [1,2]. The ESO equations are:

z1_dot = z2 - beta1*(z1 - theta)   (10)

z2_dot = z3 - beta2*(z1 - theta) + b0*delta   (11)

z3_dot = -beta3*(z1 - theta)   (12)

ESO bandwidth: omega_o = 80 rad/s. Bandwidth-parameterized gains: beta1 = 3*omega_o, beta2 = 3*omega_o^2, beta3 = omega_o^3. The control law cancels the estimated disturbance and applies PD tracking:

delta = (u0 - z3) / b0,    u0 = Kp*(theta_ref - z1) - Kd*z2   (13)

Input gain approximation: b0 = F_t / I_yy. Controller bandwidth: omega_c = 14 rad/s. Kp = omega_c^2, Kd = 2*omega_c. ADRC achieves zero overshoot across all 500 Monte Carlo runs in all three scenarios, making it the recommended flight controller.

V.  HARDWARE-IN-THE-LOOP DIGITAL TWIN ARCHITECTURE

The Project Neal digital twin is a full-stack web application enabling real-time simulation, hardware telemetry ingestion, and interactive 6-DOF visualization. This extends the original web-based telemetry dashboard [originally HTML/JS, later Figures 5.3-5.7] into a complete HIL engineering platform.

5.1  Three-Tier Architecture

Layer 1 — Simulation and API (Python 3.11 / FastAPI): Hosts the 3-DOF and 6-DOF numerical integrators, all four controller implementations, Monte Carlo batch runner, and real-time telemetry endpoints. Backend runs hardware telemetry polling at 4 Hz and simulation at request time (~50 ms for N=200 3-DOF, ~90 ms for N=300 6-DOF). Optional MATLAB engine integration invokes the Simulink nonlinear model when available.

Layer 2 — Visualization (React 18 / Three.js / @react-three/fiber): Renders the CAD-accurate OBJ/MTL rocket model with correct material assignments (white glossy body tube, red nose cone and trapezoidal fins per Project Neal livery). The scene updates at 60 fps driven by the simulation time index. Separate Canvas instances provide roll, pitch, and yaw attitude cubes on the telemetry page. Plotly.js renders all charts with tabular-numeric aerospace formatting.

Layer 3 — Hardware Interface (STM32 UART to FastAPI bridge): The STM32 flight controller streams a 45-byte binary telemetry packet at 10 Hz via UART DMA (115200 baud). The FastAPI backend decodes this and exposes it as a JSON REST endpoint polled by the frontend at 4 Hz. The launch station page drives the ARM/COUNTDOWN/RESET state machine and displays real hardware comms test results (UART, SPI MPU-6050, I2C BMP280, PWM servo channels).

| Layer | Technology | Key Function |
| --- | --- | --- |
| Simulation API | FastAPI / Python 3.11 | 3/6-DOF integration, 4 controllers, Monte Carlo, telemetry bridge |
| Frontend | React 18 / Three.js / Plotly | 6-DOF visualization, TVC joystick pad, live charts |
| Firmware | STM32F103 / HAL/CMSIS | Guidance loop 50 Hz, Kalman filter, UART DMA 10 Hz |
| Attitude Sensor | MPU-6050 (I2C 400 kHz) | Gyro + accel 200 Hz; complementary/Kalman fusion |
| Barometer | BME280 (I2C) | Altitude 25 Hz, low-pass at 5 Hz |
| Actuators | MG90 Servos x2 | TVC gimbal pitch+yaw, PWM 50 Hz, 900-2100 us |
| Telemetry | LoRa SX1276 915 MHz | RSSI > -120 dBm, range > 2 km LOS |

Table II — Project Neal Digital Twin Technology Stack

5.2  Interactive Features

The digital twin dashboard provides seven operational tabs: Mission Control (3D visualization + trajectory + pitch/altitude charts), TVC Simulator (interactive joystick pad with manual gimbal deflection, controller selection, 3-DOF/6-DOF mode toggle), Live Telemetry (attitude cubes, sensor data, history charts), Launch Station (ARM/COUNTDOWN state machine, onboard comms test, sensor validation), Vehicle Config (all physical parameters editable with live re-simulation), Benchmarks (Monte Carlo results table and comparison charts), and Research Paper (embedded interactive paper view).

VI.  SIMULATION METHODOLOGY

6.1  OpenRocket Aerodynamic Analysis

The rocket configuration was designed in OpenRocket with components including an Ogive nose cone, PVC body tube, and trapezoidal fin set. OpenRocket computes CG (63.4 cm), CP (79.1 cm), stability margin (2.06 cal), and the full flight simulation using the G74W thrust curve (peak 91.2 N, 1.12 s burn). Key simulation outputs: apogee 59.7 m, maximum velocity 31.5 m/s, time to apogee 4.12 s [26, 28].

6.2  MATLAB/Simulink Controller Validation

A 3-DOF state-space model was implemented in Simulink, with physical parameters imported directly from OpenRocket. The PID controller was tuned using manual iterative gain adjustment: proportional first (eliminate oscillation), derivative second (reduce overshoot), integral last (eliminate steady-state error). The validated gains (Kp=0.2109, Ki=0.19, Kd=0.05) were then translated to the embedded 50 Hz control loop on the STM32 hardware [29, 30].

6.3  Monte Carlo Campaign

N = 500 independent runs per controller were executed under three scenarios: (1) Nominal — all parameters at design values; (2) Wind Gust — sinusoidal roll disturbance 1.5*sin(4t)*exp(-0.8t) N·m applied; (3) Mass Uncertainty — wet mass Gaussian ±5%, CG position Gaussian ±8 mm. Random seeds were fixed across all controller comparisons for reproducibility.

VII.  RESULTS AND DISCUSSION

7.1  Controller Benchmark Results

Table III summarizes key metrics averaged over 500 Monte Carlo runs for all four controllers:

| Controller | Class | Mean RMSE theta | Overshoot | Settle Time | Peak Gimbal | Success Rate |
| --- | --- | --- | --- | --- | --- | --- |
| PID | Classic | 1.98 deg | 5.0% | 0.35 s | 3.8 deg | 98.2% |
| LQI | Optimal | 1.89 deg | 7.8% | 0.28 s | 4.1 deg | 99.1% |
| MRAC | Adaptive | 1.98 deg | 30.4% | 0.55 s | 4.9 deg | 95.4% |
| ADRC | ESO Robust | 2.26 deg | 0.0% | 0.31 s | 3.2 deg | 99.8% |

Table III — Monte Carlo Benchmark Results (N=500 per Controller)

LQI achieves the lowest mean pitch RMSE (1.89 deg) but produces 7.8% overshoot, corresponding to a transient angular exceedance of 0.39 deg beyond the 5 deg reference. Under maximum dynamic pressure at t ≈ 0.4 s, this overshoot increases the structural bending moment by approximately 12% relative to nominal. MRAC shows the highest overshoot (30.4%) during the adaptation transient, with its adaptation time (0.55 s) exceeding the useful control window for this short-burn motor.

ADRC eliminates overshoot entirely across all 500 runs and all three scenarios. The higher mean RMSE (2.26 deg, +0.37 deg vs LQI) reflects ESO convergence delay (~0.05 s) but remains well within the 5 deg gimbal authority budget. The 99.8% success rate is the best of any tested architecture, confirming ADRC as the recommended flight controller for Project Neal [1, 2, 36].

7.2  Hardware Bench Testing

The flight computer was validated through manual tilt testing at approximately 50 Hz. Physical servo response was confirmed visually; serial telemetry was monitored for correct data flow; and the web dashboard 3D orientation model was verified to track the physical sensor correctly. The Kalman filter produced smooth angle estimates with less than 0.3 deg noise floor during bench testing. LoRa telemetry link maintained data integrity at 10 Hz over 15 m bench range, with RSSI approximately -45 dBm [5.3].

7.3  6-DOF Cross-Coupling Analysis

The 6-DOF simulation reveals roll excitation under wind gust scenarios. With no dedicated roll control (single-axis TVC), roll accumulates at 0.8 deg/s during the 1.12 s burn, reaching phi_max = 0.90 deg at burnout. This is within structural limits. The yaw channel under ADRC remains stable, showing peak yaw rate below 0.4 deg/s. A future reaction control system for roll damping is recommended beyond 400 m altitude.

VIII.  HARDWARE IMPLEMENTATION

8.1  Flight Computer

The final flight computer uses an STM32F103C8T6 (ARM Cortex-M3, 72 MHz, 64 kB Flash, 20 kB SRAM). After four hardware iterations (Arduino Nano → ESP32 → Arduino Mega → STM32), the STM32 was selected for its processing power, peripheral availability, and mission-critical reliability [5.3.1].

The 50 Hz main control loop budget: IMU read via I2C DMA (MPU-6050): 0.8 ms; Kalman filter computation: 0.4 ms; PID/ADRC calculation: 0.3 ms; servo PWM update (TIM3/TIM4): 0.1 ms; LoRa telemetry + SD card log: 0.4 ms; Total: 2.0 ms (10% duty cycle at 50 Hz).

8.2  Sensor Suite

MPU-6050 (I2C, 400 kHz) — 3-axis gyroscope (±500 deg/s, 0.005 deg/s noise floor) and 3-axis accelerometer (±4g). BME280 (I2C) — barometric altitude at 25 Hz, resolution 0.12 m, filtered at 5 Hz. No magnetometer is used; yaw stabilization is limited to the propulsion phase only [8].

8.3  Telemetry System

LoRa SX1276 (915 MHz, 125 kHz BW, SF7, CR 4/5) provides a bidirectional link to the Arduino Nano ground station, which bridges serial data to the web dashboard. Packet format: pitch, roll, yaw (float × 3), altitude (float), servo outputs (int × 2), battery voltage (float), RSSI (int). The FastAPI digital twin backend also decodes the same 45-byte binary packet for real-time telemetry endpoint delivery.

| Component | Part | Interface | Role |
| --- | --- | --- | --- |
| Microcontroller | STM32F103C8T6 | — | 50 Hz guidance loop, main controller |
| IMU | MPU-6050 | I2C 400 kHz | Gyro + accel 200 Hz, Kalman fusion |
| Barometer | BME280 | I2C | Altitude 25 Hz |
| Servo x2 | MG90 digital | PWM 50 Hz | TVC gimbal pitch + yaw |
| Telemetry | LoRa SX1276 | SPI | 915 MHz, 10 Hz packet, > 2 km range |
| Storage | MicroSD | SPI | Full flight log at 50 Hz |
| Ground Station | Arduino Nano + SSD1306 | UART/SPI/I2C | Data bridge to web dashboard |

Table IV — Hardware Bill of Materials (Electronics)

IX.  CONCLUSION AND FUTURE WORK

This thesis has successfully presented the complete design, simulation, hardware prototyping, and digital twin integration of a thrust vector control system for Project Neal, a small-scale solid-propellant sounding rocket. Four control architectures were systematically benchmarked under 500-run Monte Carlo campaigns. ADRC achieved zero overshoot and a 99.8% mission success rate, confirming it as the optimal controller for the mass-transient-dominant propulsion phase where classical PID and adaptive MRAC fall short.

The hardware prototype successfully integrated an STM32 flight computer, Kalman filter sensor fusion, servo-actuated TVC gimbal, and LoRa telemetry, validated through extensive bench testing. The hardware-in-the-loop digital twin frontend provides real-time 6-DOF visualization, interactive controller simulation, live sensor monitoring, and a launch station countdown interface. This unified workflow — from OpenRocket aerodynamic design through MATLAB/Simulink controller tuning to embedded firmware to digital twin visualization — provides a replicable blueprint for low-cost TVC development in the university rocketry community.

Future work: (1) static fire test to validate 3D-printed gimbal under thermal and vibrational loading; (2) full flight test with GPS-aided Kalman filter trajectory estimation; (3) magnetometer integration for yaw authority; (4) ADRC gain auto-tuning from flight data; (5) roll reaction control system for precision trajectories above 400 m; (6) Simulink HIL closed-loop integration for automated controller certification.

REFERENCES

[1] J. Han, "From PID to Active Disturbance Rejection Control," IEEE Trans. Ind. Electron., vol. 56, no. 3, pp. 900–906, Mar. 2009.

[2] Z. Gao, "Active Disturbance Rejection Control: A Paradigm Shift in Feedback Control System Design," Proc. American Control Conf. (ACC), Minneapolis, MN, pp. 2399–2405, 2006.

[3] T. Youds, "Development of an Active Control System for a Canard Controlled Sounding Rocket," M.Sc. Thesis, May 2023.

[4] P. Kandel, M. G. Rajagopal, A. K. Yadav, S. Pandit, and N. Dev, "Design of Self-Stabilization Thrust Vector Control System (TVCs) for Rockets and Missiles," Int. Res. J. Eng. Technol., pp. 2936–2942, 2022. [Online]: www.irjet.net

[5] S. E. E. Profile, "Rising from the Ashes: The Meteoric Success of DhumketuX in the Bangladesh Tech Industry," Jul. 2023, pp. 1–3.

[6] D. Kim and A. Iturregui, "Developing a Control Algorithm and Simulation for Thrust Vector Controlled Rockets," pp. 1–30, 2022.

[7] R. Nezha, "Missile Aerodynamics and Control Systems," doctoral dissertation, pp. 1–203, Dec. 2014.

[8] H. R. Nunn, "The Jet Vane Thermal Modeling Using Parametric System Identification," NPS69-88-001, 1988.

[9] M. S. Ummah, "Tail control aerodynamics in tactical missiles," Sustain., vol. 11, no. 1, pp. 1–14, 2019.

[10] Y. Jin, X. He, J. Zhang, B. Jiang, and Z. Wu, "Experimental study on emission performance of an LPP/TVC," Chinese J. Aeronaut., vol. 25, no. 3, pp. 335–341, 2012.

[11] S. Zivkovic et al., "Experimental and simulation testing of thermal loading in jet tabs of a TVC system," Therm. Sci., vol. 20, no. suppl. 1, pp. 275–286, 2016.

[12] Y. Yang, "Spacecraft Attitude and Reaction Wheel Desaturation Combined Control Method," IEEE Trans. Aerosp. Electron. Syst., vol. 53, no. 1, pp. 286–295, 2017.

[13] Combustion Institute, "Version of Record," Combustion and Flame, Jan. 2018.

[14] H. Umasaki and Y. Mitarai, "Ammonium perchlorate composite propellants," ACS Symposium Series, 1993, 7 pp.

[15] I. Type, "Pontryagin's Maximum Principle," Encyclopedia of Control Theory, 2025.

[16] J. Doyle, K. Glover, P. Khargonekar, and B. Francis, "State-space solutions to standard H2 and H-infinity control problems," Proc. ACC, pp. 1691–1696, 1988.

[17] B. Li, X. Rui, F. Yang, and G. Wang, "Design and implementation of an adaptive Kalman filtering for the launcher of multiple launch rocket system," Int. J. Adapt. Control Signal Process., vol. 32, no. 3, pp. 447–463, 2018.

[18] F. M. Callier and C. A. Desoer, Multivariable Feedback Systems. New York: Springer-Verlag, 1982.

[19] F. Reuben, "A Robust Control Approach for Rocket Landing," M.Sc. thesis, Univ. Edinburgh, 2017.

[20] D. W. Gu, P. H. Petkov, and M. M. Konstantinov, Advanced Textbooks in Control and Signal Processing with MATLAB. Springer, 2005.

[21] W. Sun, "Research on Thrust Vector/Aerodynamics Compound Control Method for Solid Rocket Based on Proportion Mixing," Chinese Control Conf. CCC, 2020, pp. 3591–3594.

[22] G. Hartley, "H-infinity controller design for an SISO electromechanical actuator," in Proc. AIAA GNC Conf., Aug. 1992.

[23] D. E. Schinstock, D. A. Scott, and T. A. Haskew, "Modeling and estimation for electromechanical thrust vector control of rocket engines," J. Propuls. Power, vol. 14, no. 4, pp. 440–446, 1998.

[24] L. Sopegno, P. Livreri, M. Stefanovic, and K. P. Valavanis, "Thrust Vector Controller Comparison for a Finless Rocket," Machines, vol. 11, no. 3, pp. 1–15, 2023.

[25] A. B. Kisabo et al., "Fuzzy logic control for an expendable launch vehicle autopilot," Eur. J. Sci. Res., vol. 59, no. 2, pp. 226–240, 2011.

[26] S. Niskanen, "Open Rocket Technical Documentation," pp. 1–118, 2010.

[27] T. Van Milligan, Model Rocket Design and Construction, 3rd ed., 2008, p. 282.

[28] M. J. L. Caballes et al., "Rocket Trajectory Prediction using OpenRocket Simulator and Featherweight GPS Tracker," Proc. 10th Annu. SISE Conf., pp. 97819384962–1, 2021.

[29] N. Basil, H. M. Marhoon, and A. R. Ibrahim, "A new thrust vector-controlled rocket based on JOA using MCDA," Meas. Sensors, vol. 26, p. 100672, 2023.

[30] X. Fan, X. Bai, Z. Jiang, and S. Zhang, "An Optimization Method of Attitude Control Parameters Based on Genetic Algorithm for the Boost-Glide Rocket," IEEE Trans. Aerosp. Electron. Syst., vol. 60, no. 4, pp. 4356–4376, 2024.

[31] Ministry of Higher Education Bangladesh, "Attitude Control of Rocket via Thrust Vector Control Technique," 2022.

[32] A. Alvarez et al., "Design and Integration of a High-Powered Model Rocket," B.Sc. MQP, Worcester Polytechnic Institute, Mar. 2019.

[33] M. Garrison, S. Steffan, S. Tollefson, and M. Reinecke, "Thrust Vector Control (TVC) System Architecture Trade Study Overview," in Proc. AIAA, Jul. 2007, doi: 10.2514/6.2007-5848.

[34] A. Krammer, L. Blecha, and M. Lichtenberger, "Fin actuation, thrust vector control and landing leg mechanisms design for the RETALT VTVL launcher," CEAS Space J., vol. 14, no. 3, pp. 577–591, 2022.

[35] A. Bolonkin, J. Friedlander, and S. Neumann, "Retractable Kinetic Towers Against Rocket — A Deployable Active Device to Monitor and React to Threats at Borders," unpublished.

[36] C. Yang, X. Cai, L. Wu, and Z. Guo, "Addressing Launch and Deployment Uncertainties in UAVs with ESO-Based Attitude Control," Drones, vol. 8, no. 8, 2024.

[37] R. De Celis and L. Cadarso, "Neural Network-Based Controller for Terminal Guidance Applied in Short-Range Rockets," IEEE Aerosp. Electron. Syst. Mag., vol. 38, no. 4, pp. 28–42, 2023.

[38] B. L. Stevens, F. L. Lewis, and E. N. Johnson, Aircraft Control and Simulation: Dynamics, Controls Design, and Autonomous Systems, 3rd ed. Wiley, 2015.

[39] STMicroelectronics, "STM32F103x8/B Reference Manual," RM0008 Rev. 21, 2023.

[40] InvenSense, "MPU-6000 and MPU-6050 Product Specification," Rev. 3.4, 2013.

[41] Bosch Sensortec, "BMP280 Digital Pressure Sensor Data Sheet," Rev. 1.14, 2015.

[42] T. H. E. Explosive Substances Act, "The Bangladesh Code Volume VI," 1908.